|  | MINISTRY OF EDUCATION AND TRAINING |
| --- | --- |

FPT UNIVERSITY

| Report 3 |  |
| --- | --- |
| Parking Guidance System Solution |  |
| Group 1 |  |
| Group members | Trần Nguyễn Minh Trung – Team Leader – SE61496 Bùi Phú Hiệp – Team Member – SE61438 Nguyễn Đỗ Phương Huy – Team Member – SE61358 |
| Supervisor | Nguyễn Đức Lợi |
| Ext. Supervisor | N/A |
| Capstone Project Code | PGSS |

- Ho Chi Minh City, Jan, 2017

This page is intentionally left blank

# Table of Contents

Table of Contents	1

List of Tables	3

List of Figures	3

Definitions, Acronyms and Abbreviations	3

B. Software-Hardware Project Management Plan	4

1. Problem Definition	4

1.1. Name of this Capstone Project	4

1.2. Problem Abstract	4

1.3. Project Overview	4

1.3.1. Current Situation	4

1.3.2. The Proposed System	4

1.3.2.1. Interaction Block	5

1.3.2.2. Information Block	5

1.3.2.3. Central Control Unit	5

1.3.2.4. Web API Server	5

1.3.2.5. Mobile Application	5

1.3.3. Boundaries of the System	5

1.3.4. Future Plans	6

1.3.5. Development Environment	7

1.3.5.1. Hardware requirements	7

1.3.5.2. Software requirements	8

2. Project organization	8

2.1. Software Process Model	8

2.2. Roles and responsibilities	9

2.3. Tools and Techniques	10

3. Project Management Plan	11

3.1. System development life cycle	11

3.2. Plan Detail	13

3.3. All Meeting Minutes	16

4. Coding Convention	16

4.1. C/C++ Convention	16

4.2. C#, ASP.NET Convention	16

4.3. Python Convention	16

4.4. Android Convention	17

# List of Tables

Table 1: Definitions, Acronyms and Abbreviations	3

Table 2: Database requirement	7

Table 3: API Service Requirement	7

Table 4: Provide CCU Hardware	7

Table 5: Roles and Responsibilities Details	10

Table 6: Tools	10

Table 7: Techniques	11

Table 8: System Development Life Cycle	12

Table 9: System Development Detail Plan	15

# List of Figures

Figure 1: Project Block Diagram	6

Figure 2: Iterative and Incremental development	9

# Definitions, Acronyms and Abbreviations

| Name | Definition |
| --- | --- |
| PGS | Parking Guidance System |
| Parking area | An area set aside for parking vehicles, aircraft, etc. |
| Parking lot | A place inside parking area that provide space for one vehicle |
| IoT | Internet of Things |
| CCU | Central Control Unit |

Table 1: Definitions, Acronyms and Abbreviations

# C. Software – Hardware Requirement Specification

## User Requirement Specification

### Parking Guidance System

### Mobile Application

Users can see nearest parking area (as a marker on map)from his location.

Users can see empty slot and detail information about parking area by touching a marker on map.

User can reserve a parking slot.

## System Requirement Specification

### External Interface Requirement

#### User Interface

User interface of mobile app must be simple, clearly and easy to use.

The color of mobile app must be elegant, not garish.

Each UI element must be arranged logically, allowing user access easily.

#### Hardware Interface

##### Block Diagram

Figure ##: PGSS Block Diagram

##### Raspberry Pi 3

Figure ##: <Hardware>  Raspberry Pi 3

Overview: Raspberry Pi 3 is powerful mini-computer with many features. PGSS use Raspberry Pi 3 to control other hardware and send signals between hardware and mobile application.

Specification:

| SoC | Broadcom BCM2837 |
| --- | --- |
| CPU | 4× ARM Cortex-A53, 1.2GHz |
| GPU | Broadcom VideoCore IV |
| RAM | 1GB LPDDR2 (900 MHz) |
| Network | 10/100 Ethernet, 2.4GHz 802.11n wireless |
| Bluetooth | Bluetooth 4.1 Classic, Bluetooth Low Energy |
| Storage | microSD |
| GPIO | 40 |

Table ##: Raspberry Pi 3 –  Specification

##### Arduino Nano

Figure ##: <Hardware> Arduino Nano

Overview: The Arduino Nano is a small, complete, and breadboard-friendly board based on the ATmega328 (Arduino Nano 3.x).

Specification:

| Microcontroller | ATmega328 |
| --- | --- |
| Architecture | AVR |
| Operating Voltage | 5 V |
| Flash Memory | 32 KB of which 2 KB used by bootloader |
| SRAM | 2 KB |
| Clock Speed | 16 MHz |
| Analog I/O Pins | 8 |
| EEPROM | 1 KB |
| DC Current per I/O Pins | 40 mA (I/O Pins) |
| Input Voltage | 7-12 V |
| Digital I/O Pins | 22 |
| PWM Output | 6 |
| Power Consumption | 19 mA |
| PCB Size | 18 x 45 mm |
| Weight | 7 g |
| Product Code | A000005 |

Table ##: Arduino Nano – Specification

More detail about Arduino Nano:

https://www.arduino.cc/en/Main/arduinoBoardNano

##### Compass Module 3-Axis HMC5883L

Figure ##: <Hardware> Compass Module 3-Axis HMC5883L

Overview: The Compass Module 3-Axis HMC5883L is a low-field magnetic sensing device with a digital interface.

Specification:

##### RF module nRF24L01+

Figure ##:  <Hardware> RF module nRF24L01+

Overview: Reason for PGSS use RF module nRF24L01:

- Low cost

- Easy to buy

- Ultra low power consumption

Specification:

| No. | Name | I/O | Description |
| --- | --- | --- | --- |
| 1 | GND |  | Power Supply Ground |
| 2 | VCC |  | 3.3V |
| 3 | CE | I | Chip Enable |
| 4 | CSN | I | SPI Chip Select |
| 5 | SCK | I | SPI Clock |
| 6 | MOSI | I | SPI Slave Data Input |
| 7 | MISO | O | SPI Slave Data Output |
| 8 | IRQ | O | Maskable Interrupt Pin |

##### Information LED Display Module

Information LED Display Module include: 7-segment LED Display, TPIC6B595 Power Logic 8-Bit Shift Register

###### 7-segment LED Display

Figure ##: <Hardware> 7-segment LED Display

Specification:

  - 0.56 inch digit height

  - Super Red emitting color

  - White segment color, gray face

  - Low current operation

  - Easy mounting on PCB boards or sockets

###### TPIC6B595 Power Logic 8-Bit Shift Register

Figure ##: <Hardware> TPIC6B595 Power Logic 8-Bit Shift Register

Specification:

The TPIC6B595 device is a monolithic, high-voltage, medium-current power 8-bit shift register designed for use in systems that require relatively high load power.

Figure ##: TPIC6B595 Pin-outs

##### Indicator LED Module

Indicator LED Module include: Common anode RGB LED pin-out, TIP122 Transistor

###### RGB LED common anode

Figure ##: <Hardware> RGB LED common anode

Specification:

- Forward Voltage (RGB): (2.0, 3.2, 3.2)V

- Max Forward Current (RGB): (20, 20, 20)mA

- Max Luminosity (RGB): (2800, 6500, 1200)mcd

Figure ##:  Common anode RGB LED pin-out

###### TIP122 Transistor

Figure ##:  <Hardware> TIP122 Transistor

Overview:  A single digital pin on Arduino Nano do not provide enough current to power RGB LED, so we use TIP122 Transistor to provide RGB LED with power from an external source.

Specification:

- TIP122 is power transistors

- Collector Current: 5 ampere

- Collector-Emitter Volt: 100 volts

- Power Dissipation: 65 watts

#### Software Interface

- Mobile application run on Android device.

#### Communication Protocol

- Arduino Nano board communicate with Raspberry Pi 3 via RF Module.

### System Overview Use Case

Figure 1: Overview use case diagram

### List of Use Case

#### Manager Use Case

Figure 2: Manager Use case diagram

Use case specifications

| Use Case-1 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS01 | Use-case version | 1.0 |
| Use-case name | Configuration System |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - Manager Summary: - This use case allow user to change the configuration of their system. Goal: - Manager can change the information of car park, which show to the end user. Triggers: - User click on “Setting” button. Preconditions: - Mobile application is already launch. - Manager has been logged in Post Conditions: - On Success: New configuration is apply and save to server - On Failure: Show error message Main Success Scenario: No.; Actor Action; System Response. 1; User click on “Setting” button; Application navigate to “Setting” menu. 2; User select option in the Menu Change by click toggle or change value in the text box Select “Submit” button; Change the value and save to server. Alternative Scenario: - N/A Exceptions: - N/A Business Rules: - N/A |

| Use Case-2 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS02 | Use-case version | 1.0 |
| Use-case name | Manage Car Park |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - Manager Summary: - This use case allow user to change their car park info. Goal: - Manager can change the information of car park, which show to the end user. Triggers: - User select their car park. - Click “Edit” Preconditions: - Mobile application is already launch. - Manager has been logged in Post Conditions: - On Success: New information of edited car park saved to server. - On Failure: Show error message Main Success Scenario: No.; Actor Action; System Response. 1; User select car park. User click on “Edit” button; Application navigate to “Setting” menu. 2; User select option in the Menu Change by click toggle or change value in the text box Select “Submit” button; Change the value and save to server. Alternative Scenario: - N/A Exceptions: - N/A Business Rules: - N/A |

| Use Case-3 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS03 | Use-case version | 1.0 |
| Use-case name | Manage Area |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - Manager Summary: - This use case allow user to change the status of each area. Goal: - The status of selected area updated and change in mobile app. Triggers: - User select their car park. - User select area in selected car park. Preconditions: - Mobile application is already launch. - Manager has been logged in Post Conditions: - On Success: New configuration is apply and save to server - On Failure: Show error message Main Success Scenario: No.; Actor Action; System Response. 1; User select car park; Application change to car park detail page. 2; User select area in the selected car park; Application change to area detail page. 3; User select status in the drop down list. Click “Update” button; The status of the area will change on server and update in mobile application. Alternative Scenario: - N/A Exceptions: - N/A Business Rules: - N/A |

| Use Case-4 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS04 | Use-case version | 1.0 |
| Use-case name | Manage Parking Slot |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - Manager Summary: - This use case allow user to manage the parking slot. Goal: - The status of selected area updated and change in mobile app. Triggers: - User select their car park. - User select area in selected car park. - Then select parking slot Preconditions: - Mobile application is already launch. - Manager has been logged in Post Conditions: - On Success: New configuration is apply and save to server - On Failure: Show error message Main Success Scenario: No.; Actor Action; System Response. 1; User select car park; Application change to car park detail page. 2; User select area in the selected car park; Application change to area detail page. 3; User select parking slot to edit After change information, select “Update” button; The information of parking slot is change on server and update in mobile application.. Alternative Scenario: No.; Actor Action; System Response. 1; User select car park; Application change to car park detail page. 2; User select area in the selected car park; Application change to area detail page. 3; User click menu beside list parking spot to delete.; The parking spot will be set to deleted in server and update in mobile app.. Exceptions: - N/A Business Rules: - N/A |

#### Administrator Use Case

| Use Case-5 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS05 | Use-case version | 1.0 |
| Use-case name | Add Car Park |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - Administrator Summary: - This use case allow user to add new car park to the system Goal: - New car park is added and save to server. Triggers: - User click on “Add” button. Preconditions: - Mobile application is already launch. - Administrator has been logged in Post Conditions: - On Success: New car park is save to server - On Failure: Show error message Main Success Scenario: No.; Actor Action; System Response. 1; User click on “Add” or “+” button; Application navigate to add car park menu. 2; User fill in the textbox Select “Submit” button; New car park with filled in info is added to server. Alternative Scenario: - N/A Exceptions: - Name of the car park is unique - Address of the car park is unique (don’t has same latitude and longitude) Business Rules: - N/A |

#### End User Use Case

| Use Case-6 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS06 | Use-case version | 1.0 |
| Use-case name | Check number of empty slot |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - End User Summary: - This use case allow user view number of empty slot in each car park Goal: - Show number of empty slot Triggers: - User login to the mobile application Preconditions: - Mobile application is already launch. - End user had logged in. Post Conditions: - On Success: User know the number of empty slot in car park - On Failure: Don’t show number of empty slot in car park Main Success Scenario: No.; Actor Action; System Response. 1; User log in to the application; Show the map with the marker as car park and the number, which indicate the number of empty slot. Alternative Scenario: - N/A Exceptions: - The number will have tick/ exclamation points to show that the number is recently update or not. Business Rules: - Tick: recently update - Exclamation points: number is not update in more than 1 hour. |

| Use Case-7 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS07 | Use-case version | 1.0 |
| Use-case name | Book parking slot |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - End User Summary: - This use case allow user to book parking slot before go to the car park Goal: - Book the parking slot before go to car park Triggers: - User has selected the car park to book Preconditions: - Mobile application is already launch. - End user had logged in. Post Conditions: - On Success: User book the parking slot success - On Failure: Show error message when book Main Success Scenario: No.; Actor Action; System Response. 1; User log in to the application; Show the map with the marker as car park and the number, which indicate the number of empty slot. 2; User select the car park they want to book; Show the “Book” button if has empty slot. 3; Fill information for transaction Click “Submit”; Make a transaction and set one parking slot to booked Show the address of booked parking slot to the user. Alternative Scenario: - N/A Exceptions: - Transaction fail by 3rd party. Business Rules: - N/A |

| Use Case-8 specification |  |  |  |
| --- | --- | --- | --- |
| Use-case no. | PGSS08 | Use-case version | 1.0 |
| Use-case name | Search car park |  |  |
| Author | Bui Phu Hiep |  |  |
| Date | 13/02/17 | Priority | High |
| Actor: - End User Summary: - This use case allow user to search a car park by name or address Goal: - Show the searched car park Triggers: - User login to the mobile application Preconditions: - Mobile application is already launch. - End user had logged in. Post Conditions: - On Success: Show the searched car park on the map if success - On Failure: Show message error Main Success Scenario: No.; Actor Action; System Response. 1; User log in to the application; Show the map with the marker as car park and the number, which indicate the number of empty slot. 2; Enter the name or address in the search bar Press “Enter” or click “Search”; Find the car park base on name or address then focus on the map.. Alternative Scenario: No.; Actor Action; System Response. 1; User log in to the application; Show the map with the marker as car park and the number, which indicate the number of empty slot. 2; Enter the name or address in the search bar Press “Enter” or click “Search”; Show message don’t have car park if the name or address is incorrect. Exceptions: - N/A Business Rules: - N/A |

## Software System Attribute

### Usability

- User controls all system components via only mobile application.

- The system can install easily.

- User can learn how to use the system fast.

### Reliability

### Availability

- The mechanical component require electrical system to work well.

- Hardware components are easy to find in the market.

### Security

- Mobile application require authentication and authorization implement well because manager and end user use the same application.

### Maintainability

- Use plug and play component so we can easily replace it.

### Portability

- Easy to construct.

### Performance

- Detection car is fast, less then 50ms.

- The speed of server can scale base on the budget easily.

## Conceptual Diagram

Figure 3: Conceptual Diagram

Data Dictionary

| Entity Data dictionary: describe content of all entities |  |
| --- | --- |
| Entity Name | Description |
| CarPark | Descript all car park information in the system |
| Area | Describe all area detail in car park |
| ParkingLot | Describe parking lot information in the area |
| Item | Describe hardware item in each parking lot |
| BookingHistory | Describe the booking history of the user |
| Transaction | Save the transaction of each booking |